Diagnostics by nature can be quite repetitive, and am tempted to say boring; the obvious danger is omission or complacency in your process. This risk becomes greater with experience, the temptation to pre judge or take short cuts is always there.

Sometimes the challenge is down to incomplete information, or the dreaded been worked on before, often adding to or masking the original symptoms.

With this in mind a particular challenge came our way quite recently.

Bmw 318i/e91 eng/n46b20b

Courtesy of a good retail customer Arnold Clarke group, this vehicle suffered a misfire symptom, especially from cold.

The previous history was reported in quite some detail, previous to our involvement, it had been established that this vehicle had a long history of work carried out by the local bmw dealership without success.

The vehicle had recorded misfire events with #4 cly,there was also a decernable timing chain rattle on cold start up although not severe.new coils and sparking plugs had been fitted.we began with a comprehensive serial data examinatrion , focusing on live data,some of you may have noticed how poor dealership level live data has become!

The vehicle ran remarkably well at idle , only displaying the misfire under severe load.

Possible faults fall into 3 main categories

1/fuelling errors

2/ignition malfunction

3/mechanical malfunction

All quite straight forward 20 years ago,however the n46

Engine offers more of a challenge, with variable valve timing”vanos”,and difficult component access to say the least.

We began by examining the spark plugs , they were quite sooty, soft soot common with poor combustion properties,we opted to apply a thorough bg flush process both in the oil and intake system ,suspecting sticing valves or tappets.

Replacing the oil and re assessing the engine did show an improvement but the underlying problem persisted.

During our examination of the ignition system with the Pico scope we noted a wiring problem @ #4 coils, wiggling the loom induced a definite misfire, job done? Well not quite.

Further testing with the scope it was noted that the vanos was slightly out of position, coupled with the timing chain noise we advised it be replaced.

Following its replacement a further improvement was gained, when at normal operating temperature the engine ran quite well. Exhaust gas analysis was perfect, even on acceleration he’s rose to approx 250ppm.

However from cold, load acceleration still produced a misfire?

Conventional compression testing failed to produce sufficient evidence or confidence to strip the engine.

Due to the lack of live data we were forced to obtain a donor vehicle for comparative testing, this came in the guise of a z4 with an identical engine.

All electronic data matched perfectly.

Ignition burn profile, injector current path, frequency, valve timing position, and even lambda sensor evaluation, all failed to provide any further evidence.

Common sense at this point would surges an engine mechanical problem, wouldn’t it?

This is the point I made earlier about 20 years ago,

It’s at this point I focused on the criteria required to monitor the problem. A cold engine with a narrow window of opportunity, the engine not only running but on load, and monitoring functionality in real time!

Back to the Pico I think, firstly I had to establish best criteria from a good known sample the z4.

I decided to conduct a compression test in real time with the engine running, the initial evaluation included a crank, start, run, and acceleration profile of the engine compression performance.

Waveform 1

Note the overall profile and compression value expressed in voltage using the Pico pressure transducer.

Next we applied the same test in identical conditions to the 318i,

Waveform 2

Note similar crank start and idle values , reinforcing earlier test results , however on acceleration note a much lower cylinder pressure, and more significant the compression loss on full load.

Waveform close up 3

The remarkable thing is that the compression anomalies can occur on individual piston strokes, all fuelling and ignition had of course been removed from the test cylinder, leaving only mechanical functionality responsible for the results.

So there we have it, a internal mechanical problem, what restrained us in forming a decision sooner was a lack of absolute proof, not a bad thing , but also the cost implications , if we got it wrong !

Caution Vis common sense.

The experience and processes developed in our workshop are then built into all  our training programmes , all subjects are covered, choice of tools, process and risk assessment form part of the programme , but the most valuable and unique asses is hands on experience over 40 years.

Contact Annette at ads for details 01772 201597

enquries@ads-global.co.uk

AUTOiNFORM .co.uk